National Textile University, Faisalabad

Department of Computer Science

| Name: | Amna Khalid |
| --- | --- |
| Class: | BSCS_A 5th Semester |
| Registration No: | 23-NTU-CS-1016 |
| Assignment : | Project |
| Course Name: | Embedded IOT |
| Submitted To: | Sir Nasir |
| Submission Date: | 26-10-2025 |

Project Documentation: Multi-Task ESP32 Control System

1. Project Overview

This project implements two distinct tasks—LED mode cycling (Task A) and press-type detection (Task B)—on a single ESP32 Dev Module. The system is controlled by two pushbuttons and provides visual feedback via an OLED display and audible feedback via a buzzer.

| Component | Quantity | Purpose |
| --- | --- | --- |
| ESP32 Dev Module | 1 | Microcontroller running the logic. |
| OLED Display (SSD1306, I²C) | 1 | Displays the current mode and button event. |
| Push Button | 2 | Inputs: one for mode cycle/reset, one for press detection. |
| LED (Red, Green, Blue) | 3 | Visual outputs for mode sequences and toggling. |
| Buzzer | 1 | Audible output for the long-press event. |

TASK A:

Circuit Diagram: Design a Wokwi circuit and draw a neat hand-sketch including:

• 2 push buttons

• 3 LEDs

• 1 buzzer

• 1 OLED

Task A — Coding: Use one button to cycle through LED modes (display the current state on

the OLED):

1. Both OFF

2. Alternate blink

3. Both ON

4. PWM fade

Use the second button to reset to OFF.

TASK A  :FIXED BUTTON ISSUE

WOKWI LINK  :

https://wokwi.com/projects/445872855716310017

SCREENSHOT:

Both OFF:

Alternate blink:

Both ON:

PWM fade:

Loom Link Video Link:

https://www.loom.com/share/408e4a6b82844adfab2b7d0995c2d7a7

Code Of The Program:

//name:Amna khalid

//reg no:23-NTU-CS-1016

//TASK 1:1. Both OFF

//2. Alternate blink

//3. Both ON

//4. PWM fade

//Use the second button to reset to OFF.

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define LED1 25

#define LED2 27

#define LED3 26

#define BTN_MODE 32

#define BTN_RESET 33

Adafruit_SSD1306 display(128, 64, &Wire, -1);

int mode = 1;

void showMode() {

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(WHITE);

display.setCursor(0, 10);

display.print("Mode: ");

display.print(mode);

if (mode == 1) display.print(" OFF");

if (mode == 2) display.print(" ALT");

if (mode == 3) display.print(" ON");

if (mode == 4) display.print(" FADE");

display.display();

}

void setup() {

pinMode(LED1, OUTPUT);

pinMode(LED2, OUTPUT);

pinMode(LED3, OUTPUT);

pinMode(BTN_MODE, INPUT);

pinMode(BTN_RESET, INPUT);

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

showMode();

}

void loop() {

if (digitalRead(BTN_RESET) == HIGH) {

mode = 1;

showMode();

}

if (digitalRead(BTN_MODE) == HIGH) {

mode++;

if (mode > 4) mode = 1;

showMode();

delay(300);

}

//

if (mode == 1) {

digitalWrite(LED1, LOW);

digitalWrite(LED2, LOW);

digitalWrite(LED3, LOW);

}

if (mode == 2) {

digitalWrite(LED1, HIGH);

digitalWrite(LED2, LOW);

delay(300);

digitalWrite(LED1, LOW);

digitalWrite(LED2, HIGH);

delay(300);

}

if (mode == 3) {

digitalWrite(LED1, HIGH);

digitalWrite(LED2, HIGH);

digitalWrite(LED3, HIGH);

}

if (mode == 4) {

for (int i = 0; i < 255; i++) {

analogWrite(LED3, i);

delay(5);

}

for (int i = 255; i > 0; i--) {

analogWrite(LED3, i);

delay(5);

}

}

}

HANDMADE SKETCH:

TASK B:

— Coding: Use a single button with press-type detection (display the event on the OLED): • Short press → toggle LED • Long press (> 1.5 s) → play a buzzer tone.

WOKWI LINK:

https://wokwi.com/projects/445878489469458433

WORKING SCREENSHOT:

Buzzer running:

Loom video link:

https://www.loom.com/share/69575515d16d42e2ae0780c8dac4d495

Code of the program:

// NAME: AMNA KHALID

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

// ---------- OLED CONFIG ----------

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

// If your OLED has a reset pin, replace -1 with the pin number

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

// ---------- PIN DEFINITIONS ----------

#define BUTTON 32

#define LED 25

#define BUZZER 14

// ---------- GLOBAL VARIABLES ----------

bool ledState = false;

unsigned long pressStart = 0;

bool isPressed = false;

// ---------- FUNCTION PROTOTYPES ----------

void playMelody();

void shortBeep();

void showMessage(String msg);

// ---------- SETUP ----------

void setup() {

Serial.begin(115200);           // For debug

pinMode(BUTTON, INPUT_PULLUP);  // button connected to GND

pinMode(LED, OUTPUT);

pinMode(BUZZER, OUTPUT);

Wire.begin();

// Initialize OLED

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println("SSD1306 allocation failed");

for (;;); // stop here if OLED fails

}

display.clearDisplay();

display.setTextColor(SSD1306_WHITE);

display.setTextSize(1);

display.setCursor(0, 0);

display.println("System Ready...");

display.display();

delay(1000);

showMessage("Waiting for button...");

}

// ---------- MAIN LOOP ----------

void loop() {

int buttonState = digitalRead(BUTTON);

// Detect button press

if (buttonState == LOW && !isPressed) {

isPressed = true;

pressStart = millis(); // time press began

}

// Detect button release

if (buttonState == HIGH && isPressed) {

isPressed = false;

unsigned long pressDuration = millis() - pressStart;

if (pressDuration > 1500) {

// Long press -> play melody

playMelody();

showMessage("Long Press -> Melody");

} else {

// Short press -> toggle LED + short beep

ledState = !ledState;

digitalWrite(LED, ledState);

shortBeep();

showMessage("Short Press -> LED " + String(ledState ? "ON" : "OFF"));

}

}

}

// ---------- FUNCTIONS ----------

void playMelody() {

// Simple 3-tone melody for long press

tone(BUZZER, 500, 200);   // low tone

delay(250);

tone(BUZZER, 800, 200);   // medium tone

delay(250);

tone(BUZZER, 1200, 300);  // high tone

delay(350);

noTone(BUZZER);

}

void shortBeep() {

// Quick beep for short press

tone(BUZZER, 1000, 100);

delay(150);

noTone(BUZZER);

}

void showMessage(String msg) {

display.clearDisplay();

display.setCursor(0, 0);

display.println("Button Event:");

display.setCursor(0, 20);

display.println(msg);

display.display();

}

Handmade sketch:

GitHub link:

https://github.com/Amnaworkspace/embedded_iOT_1016_Amna.git